SentinelAI

Rapid Crisis Detection Platform

Product Requirements Document

| Version | 1.0 — Initial Draft |
| --- | --- |
| Status | In Review |
| Target Platform | Jetson Nano / Raspberry Pi 5 |
| Crisis Scope (v1) | Fire · Earthquake · Stampede |

# 1. Executive Summary

SentinelAI is an edge-deployed crisis detection and response platform designed for venues, public spaces, and critical infrastructure. The system combines AI-driven multi-modal sensing with an agentic notification layer to detect crises such as fire, earthquake, and stampede in real time — and automatically dispatch alerts to the right authorities before a human operator would manually notice the event.

The platform runs entirely on low-cost edge hardware (NVIDIA Jetson Nano or Raspberry Pi 5 + Coral USB TPU), making it deployable without cloud dependency. A web dashboard gives authorities real-time situational awareness, and a location-aware mobile app lets bystanders report incidents and receive evacuation guidance.

Core value proposition Detect in seconds, dispatch in milliseconds, guide in real time — without cloud latency or recurring SaaS costs.

# 2. Problem Statement

Current crisis detection in most venues relies on single-sensor triggers (smoke detectors, manual pull-stations) with no AI-assisted confirmation, high false positive rates, and slow manual escalation chains. This creates three systemic failures:

- Detection lag — a fire that begins in a blind spot of a standalone smoke detector may not trigger an alarm for 3–8 minutes. AI-based vision + sensor fusion can reduce this to under 30 seconds.

- Alert latency — even when a detector fires, notification to external emergency services typically requires a human operator to make a call. Automated dispatch agents eliminate this bottleneck.

- No bystander loop — occupants have no channel to report what they see, and no way to receive real-time evacuation guidance specific to their location inside the venue.

# 3. Goals and Non-Goals

## 3.1 Goals

- Detect fire, earthquake, and stampede with >90% precision and <5% false positive rate in controlled venue conditions.

- Alert registered authorities within 10 seconds of a confirmed crisis event.

- Provide real-time evacuation routing via the authority dashboard and bystander app.

- Run fully on-device at the edge with no required cloud connectivity.

- Support extensible crisis type plugins so new detection modules (gas leak, flood, medical) can be added without refactoring the core pipeline.

- Enable bystander manual reporting as a secondary signal source that can upgrade detection confidence.

## 3.2 Non-Goals (v1)

- This is not a cloud-native SaaS product. No multi-tenancy, no hosted inference, no per-seat billing in v1.

- This does not replace or interface with official emergency dispatch systems (CAD/911 PSAP) in v1 — alerts go to registered venue authorities only.

- The system does not perform forensic video analysis or long-term video retention.

- No biometric identification of individuals is performed.

# 4. Users and Stakeholders

| User Type | Description | Primary Interface |
| --- | --- | --- |
| Venue Authority | Security leads, facility managers who monitor the dashboard and acknowledge alerts | Web Dashboard |
| Emergency Services | Fire dept, police, medical — receive automated SMS/webhook notifications on event dispatch | SMS / Webhook |
| Bystander / Occupant | Anyone physically present at the venue who has installed the app or enters the geofenced area | Mobile App |
| System Admin | Technical staff who configure sensors, cameras, floor plans, and alert trees | Admin Panel (Dashboard) |
| Hardware Operator | Person who installs and maintains the edge node, sensors, and cameras | SSH / Local UI |

# 5. System Architecture

SentinelAI is structured as five logical layers that run on the edge node, with the dashboard and mobile app as external consumers.

## 5.1 Layer 1 — Sensing

All physical input devices connected to the edge node via USB, CSI, I2C, SPI, or UART:

| Component | Interface | Crisis Relevance |
| --- | --- | --- |
| IP / USB Camera (1–4) | USB3 / CSI-2 | Fire smoke, crowd density, stampede motion |
| MPU-6050 / ADXL345 | I2C | Earthquake seismic vibration (P-wave) |
| MQ-2 Gas Sensor | GPIO / ADC | Fire — LPG, smoke, CO |
| MLX90614 IR Thermometer | I2C | Fire — surface temperature anomaly |
| DHT22 Temp/Humidity | GPIO | Fire context (dry air = elevated risk) |
| USB Microphone Array | USB Audio | Stampede panic screaming, shattering |
| BMP280 Barometer | I2C | Earthquake structural pressure drop |
| BLE Beacon (optional) | Bluetooth LE | Occupant location for app geofencing |

## 5.2 Layer 2 — AI Inference Engine

On-device inference pipeline targeting Jetson Nano (TensorRT INT8) or Raspberry Pi 5 + Coral USB TPU (TFLite INT8). All models run below 150ms latency per frame.

### Vision module

- Model: YOLOv8-nano fine-tuned on fire/smoke detection (FLAME dataset) + crowd density estimation (ShanghaiTech dataset).

- Input: 640×640 frames at 15 FPS from each camera.

- Outputs: fire confidence score (0–1), smoke confidence score (0–1), crowd density map, optical flow variance.

### Sensor fusion module

- Lightweight gradient boosted classifier (XGBoost) taking: accelerometer FFT features, gas sensor PPM, surface temp delta, barometric delta, humidity.

- Outputs: earthquake probability, fire sensor confidence.

### Audio classifier

- Model: YAMNet or PANNs fine-tuned for panic screaming, crowd roar, breaking glass, structural collapse sounds.

- Input: 1-second audio windows at 16kHz, processed every 500ms.

- Outputs: audio panic score (0–1).

## 5.3 Layer 3 — Crisis Classifier

A multi-modal fusion layer that combines outputs from all inference modules into a single crisis event probability. Uses a hand-tuned threshold matrix per crisis type to minimize false positives:

| Crisis Type | Required Signals (AND) | Supporting Signals (OR boost) | Trigger Threshold |
| --- | --- | --- | --- |
| Fire | Vision fire/smoke ≥ 0.6 | Gas sensor + IR delta OR audio | 2 of 3 signals active |
| Earthquake | Accelerometer P-wave FFT ≥ 0.7 | Barometric drop + audio | Seismic mandatory + 1 other |
| Stampede | Crowd density ≥ threshold + optical flow variance ≥ threshold | Audio panic score ≥ 0.6 | Vision both + optional audio |

The output is a structured crisis event object:

{ crisis_type, confidence, location_id, zone, timestamp, sensor_snapshot, camera_frame_url, triggered_by[] }

## 5.4 Layer 4 — Agentic Orchestration

Four parallel agents consume every confirmed crisis event object and execute their responsibilities independently:

### Dispatch Agent

- Maintains a priority-ranked notification tree per venue: SMS (Twilio / MSG91) → webhook to authority API → email escalation.

- Implements acknowledgment tracking: if no acknowledgment received within N seconds, escalates to the next tier automatically.

- Retries on connectivity failure using an exponential backoff queue with local SQLite persistence.

### Routing Agent

- Loads the venue floor plan as a weighted adjacency graph. On crisis event, computes shortest safe exit paths using Dijkstra, excluding zones proximate to the crisis location.

- Triggers MQTT messages to digital signage actuators and IP speaker systems at exit nodes.

- For earthquake events, overrides with shelter-in-place instructions to all actuators.

### Broadcast Agent

- Maintains a registry of bystander app device tokens for each venue geofence.

- Pushes geofenced Firebase Cloud Messaging (FCM) notifications to all registered devices within the active venue polygon.

- Notification payload includes: crisis type, recommended action, and deeplink to live evacuation map in app.

### Logging Agent

- Writes every event, sub-event, and agent action to a structured append-only log in PostgreSQL with PostGIS location tagging.

- Captures camera frame snapshots at event trigger time to S3-compatible local storage (MinIO).

- Provides a queryable event timeline API for the dashboard's replay viewer.

## 5.5 Layer 5 — External Interfaces

### Authority Web Dashboard

- React 18 + Vite SPA served from the edge node's nginx reverse proxy.

- Live map view: Leaflet.js with venue floor plan overlay, real-time event markers and zone heat overlays.

- Real-time sensor feed: WebSocket stream of confidence scores, sensor readings, and AI outputs.

- Alert management: acknowledge events, add notes, escalate to external agencies.

- Post-incident replay: scrub the event timeline with synchronized camera frame and sensor data.

- Admin panel: configure sensors, cameras, floor plans, alert trees, and occupancy thresholds.

### Bystander Mobile App (React Native + Expo)

- Geofence-based auto-activation: silently subscribes to the nearest registered venue when the user enters its polygon. No manual check-in required.

- Receives push notifications for AI-detected crises with deeplink to in-app evacuation map.

- Incident reporting UI: maximum 2-tap SOS flow for pre-defined report types (I am stuck, Someone needs help, I see fire/smoke, Other).

- Reports feed back into the crisis classifier as a human signal — sufficient reports can upgrade a borderline confidence score to a confirmed event.

- Live evacuation guidance screen showing safe exit paths updated in real time.

# 6. Functional Requirements

| ID | Module | Requirement | Priority |
| --- | --- | --- | --- |
| FR-01 | Inference | System must process camera frames at minimum 15 FPS per connected camera without dropping frames. | P0 |
| FR-02 | Inference | Inference engine must run fully offline. No network call shall be required for detection. | P0 |
| FR-03 | Classifier | Crisis event must not be dispatched below a configurable minimum confidence threshold (default 0.75). | P0 |
| FR-04 | Dispatch | System must send SMS/webhook alert to primary authority contact within 10 seconds of crisis confirmation. | P0 |
| FR-05 | Dispatch | Unacknowledged alerts must auto-escalate to secondary contact after a configurable timeout (default 60s). | P0 |
| FR-06 | Dashboard | Live sensor feed must update at minimum every 2 seconds via WebSocket. | P0 |
| FR-07 | Dashboard | Venue floor plan overlay must show event zone with color-coded severity. | P1 |
| FR-08 | App | Geofence activation must occur within 30 seconds of entering the venue polygon. | P1 |
| FR-09 | App | SOS report flow must be completable in 2 taps or fewer from the home screen. | P0 |
| FR-10 | App | Evacuation map must reflect routing agent updates within 5 seconds. | P1 |
| FR-11 | Logging | All crisis events, sub-events, and agent actions must be persisted before any notification is dispatched. | P0 |
| FR-12 | Extension | Adding a new crisis type must not require changes to the agent, dashboard, or app codebases — only a new inference module and classifier config entry. | P1 |

# 7. Non-Functional Requirements

| Requirement | Target / Constraint |
| --- | --- |
| Detection latency | < 30 seconds from event onset to confirmed crisis object |
| Alert dispatch latency | < 10 seconds from confirmed event to first SMS/webhook |
| Vision inference throughput | >= 15 FPS per camera at 640×640, all models combined |
| False positive rate | < 5% in representative venue test conditions |
| Edge node power consumption | < 15W average (Jetson Nano); < 25W with Coral TPU on Pi 5 |
| Offline operation | Full detection, classification, dispatch, and logging must function with no internet |
| Dashboard concurrent users | Supports >= 10 concurrent authority users without performance degradation |
| App install size | < 30 MB on both iOS and Android |
| Data privacy | No biometric data stored. Camera frames retained max 72 hours, overwritten on loop. |
| Connectivity resilience | Edge node maintains detection and local alert queue for up to 24 hours without uplink; syncs on reconnect |

# 8. Technology Stack

## 8.1 Edge Node

| Layer | Technology | Notes |
| --- | --- | --- |
| OS | Ubuntu 22.04 LTS (Jetson) / Raspberry Pi OS 64-bit | Headless server mode |
| Vision inference | PyTorch → ONNX → TensorRT (Jetson) / TFLite (Pi + Coral) | INT8 quantization for 3–5× speedup |
| Sensor pipeline | Python + smbus2, RPi.GPIO, pyserial | Async polling with asyncio |
| Audio inference | YAMNet / PANNs via TFLite | 1s windows, 500ms stride |
| ML fusion | XGBoost (sensor fusion) + rule engine (thresholds) | Serialized .pkl model |
| Message broker | Mosquitto (MQTT) | QoS 1 for actuator commands |
| Agent runtime | Python + FastAPI (internal HTTP) | Each agent = async task |
| Database | PostgreSQL 15 + PostGIS | Event log, occupancy, venue config |
| Object store | MinIO (S3-compatible) | Camera frame snapshots |
| Reverse proxy | nginx | Routes to dashboard, API, MinIO |
| Notifications | MSG91 (India SMS) / Twilio fallback + FCM | Twilio for international fallback |

## 8.2 Web Dashboard

| Component | Technology |
| --- | --- |
| Framework | React 18 + Vite + TypeScript |
| Map / floor plan | Leaflet.js with custom tile layer for venue floor plan SVG overlay |
| Real-time feed | WebSocket (native browser API, JSON messages from edge API) |
| Charts / telemetry | Recharts — sensor confidence time series |
| Styling | Plain CSS modules (no Tailwind, no component libraries) |
| Auth | JWT with short-lived tokens, stored in httpOnly cookies |

## 8.3 Mobile App

| Component | Technology |
| --- | --- |
| Framework | React Native + Expo (managed workflow) |
| Geofencing | expo-location with background geofencing task |
| Push notifications | Expo Notifications + FCM (Android) + APNs (iOS) |
| BLE beacon scanning | expo-modules-core + react-native-ble-plx (optional) |
| Map in app | react-native-maps with venue floor plan overlay |
| State management | Zustand (lightweight global store) |

# 9. Key Data Flows

## 9.1 Crisis detection flow

- Camera frames + sensor readings captured simultaneously by sensing layer.

- Vision model, sensor fusion model, and audio classifier run in parallel.

- Crisis classifier combines outputs. If combined confidence exceeds threshold, a crisis event object is created and written to PostgreSQL (logging agent acts first).

- Dispatch, routing, and broadcast agents consume the event object in parallel.

- Dispatch agent sends SMS/webhook. Routing agent triggers actuators and updates dashboard. Broadcast agent pushes FCM to bystander devices.

- Dashboard WebSocket channel pushes the new event to all connected authority clients.

## 9.2 Bystander report flow

- User taps report type in app (2-tap max). App POSTs to edge node /report API with: report_type, user_location, timestamp, device_id.

- Edge node accumulates reports per venue. If report count for a crisis type exceeds configurable threshold within a time window, confidence booster is added to the classifier.

- If boosted confidence crosses the crisis threshold, normal detection flow triggers from step 3 above.

# 10. Extensibility Architecture

Every crisis type in SentinelAI is a plugin registered in a central crisis_registry.yaml. Adding a new crisis type requires:

- A new inference module (Python class implementing the InferenceModule interface) that returns a typed confidence dict.

- A classifier config entry in crisis_registry.yaml specifying required signals, supporting signals, and trigger thresholds.

- A report_types entry for the mobile app report UI.

- An optional actuator_actions entry for the routing agent.

No changes are required to the agent layer, dashboard, database schema, or mobile app. The planned v2 crisis types include:

| Crisis Type | Key New Sensors / Models Required |
| --- | --- |
| Gas leak | MQ-5 methane sensor + ventilation actuator integration |
| Flood / water intrusion | Capacitive water sensor + ultrasonic level sensor |
| Medical emergency | Person-down pose estimation via vision model (MediaPipe) |
| Structural failure | MEMS strain gauge + acoustic emission sensor |
| Power outage | AC voltage monitor — triggers backup lighting commands |

# 11. Milestones and Delivery Plan

| Milestone | Target | Deliverables |
| --- | --- | --- |
| M0 — Hardware setup | Week 1–2 | Edge node OS, camera feeds, all I2C/GPIO sensors wired and reading |
| M1 — Inference baseline | Week 3–5 | Vision model running at target FPS, sensor fusion pipeline reading, audio classifier integrated |
| M2 — Crisis classifier | Week 6–7 | Fusion layer tuned, all three crisis types triggering correctly in test scenarios |
| M3 — Agent layer | Week 8–10 | All four agents running, SMS dispatch verified, MQTT actuator commands tested |
| M4 — Dashboard v1 | Week 11–13 | Live map, sensor feed, alert acknowledgment, basic admin panel |
| M5 — Mobile app v1 | Week 14–16 | Geofence activation, push alerts, 2-tap SOS report, live evac map |
| M6 — Integration test | Week 17–18 | End-to-end fire/earthquake/stampede drills, false positive tuning, load test |
| M7 — v1 Release | Week 19 | Deployed hardware, production dashboard, published app builds |

# 12. Risks and Mitigations

| Risk | Severity | Mitigation |
| --- | --- | --- |
| High false positive rate causing alert fatigue | High | Multi-modal fusion mandatory; tune thresholds on real venue test data before deployment |
| Jetson Nano insufficient for 4 simultaneous camera streams | Medium | Benchmark early; fallback to Pi 5 + Coral TPU or reduce to 2 cameras with wider FOV lenses |
| Cellular / WiFi outage blocking SMS dispatch | High | Local alert queue with SQLite persistence + 4G LTE fallback modem (Quectel EC25) |
| Bystander app not installed by most occupants | Medium | System is fully functional without app — it is additive signal only, not required for core detection |
| Stampede detection confusing crowded concert with panic | High | Require both density AND optical flow variance thresholds; audio as mandatory third signal in high-density venues |

# 13. Open Questions

- Should the authority dashboard support multi-venue management (multiple edge nodes aggregated to one dashboard) in v1 or defer to v2?

- What is the threshold for bystander report volume before they influence the classifier? Needs testing data.

- Does the mobile app require user authentication or anonymous reporting? Anonymous reduces friction but complicates spam protection.

- For stampede detection in Indian venue contexts (melas, railway stations), what are realistic crowd density baselines? YOLOv8 crowd models are trained primarily on Western datasets.

- Integration with government emergency services (112 India) — is a direct API available or does it require manual authority handoff?

# Appendix A — Glossary

| Term | Definition |
| --- | --- |
| Edge node | The Jetson Nano or Raspberry Pi device co-located with cameras and sensors at the venue |
| Crisis event object | Structured data payload created by the classifier that represents a confirmed crisis detection |
| TensorRT | NVIDIA SDK that optimizes deep learning models for inference on Jetson hardware |
| Optical flow variance | A measure of the directional disorder in motion vectors across a video frame — high variance indicates chaotic crowd movement |
| P-wave | The primary compressional seismic wave that arrives first during an earthquake and is detectable by accelerometers |
| MQTT | Lightweight publish/subscribe messaging protocol used for IoT device communication |
| FCM | Firebase Cloud Messaging — Google's cross-platform push notification service |
| PostGIS | A spatial extension for PostgreSQL enabling geographic queries on location-tagged data |
| MSG91 | Indian SMS gateway service with robust delivery across Indian telecom networks |

— End of Document —